Q1.1

ld  	r3, r0, 05

add 	r1, r3, r0

Q1.2

add 	r3, r3, 02

add 	r2, r2, 01

add 	r1, r2, r3

Q1.3

add 	r3, r3, r0,

st  	r3, r2, 00

Q1.4

ld 	r1, r0, 00

Q1.5

ldr 	r1, r3, 00

Q1.6

add 	r3, r1, r0

beqz	 -   r3, 04

Q1.7

jr 	 -     -   r1

Q1.8

addi 	r2, r0, 05

beqz 	  -   r2, 0C

subi 	r2, r2, 01

j 	 -     -   f4

Q2 (i) – When ALU forwarding is enabled the number of cycles is 10 and r1 finishes with 15. There is two phase clocking and pipeline forwarding where the previous two instructions can be forwarded to the ALU inputs from the ALUOUT0 & ALUOUT1 pipeline registers before the results are written back to the register file. Therefore there are no stalls in waiting for the previous instruction result to be written back.

Q2(ii) – When ALU forwarding is disabled but the CPU dependency is enabled the number of cycles is 18 and the r1 finishes with 15. There is no pipeline forwarding but there is an intelligence that tells the program to stall until the correct register value is written back.

Q2(iii) – When ALU forwarding is disabled as well as its CPU dependency the number of cycles is 10 and r1 finishes with 6. There is no pipeline forwarding and no intelligence that tells the program to stall until the correct register value is written back so therefore it gets back the wrong answer and the same number of cycles as part (i).

Q3(i) – The number of cycles is 50 and the number of instructions executed is 38. The number of instructions executed isn’t the same as the number of cycles because there are stalls each time in the loop where ldr r2, r0, 00 because it has to access memory in order to update r2 so that the next instruction can use it. There is also one stall whenever the branch prediction predicts incorrectly which happens twice at the exit of each loop.

Q3(ii) – The number of cycles goes up to 53. This is because the stalls avoided using branch prediction are gone. Now the program stalls whenever the branch is taken.

Q3(iii) – The number of cycles goes down to 46. This is because every time in the loop there is a stall when ldr r2, r0, 00 and the next instruction uses r2 and has to wait for memory access to update r2. If the instructions are switched there is no change in the output and the stall is removed with a viable instruction.